Prefabricated Living-Box

- Introduction

- Transport

- Concept of living

- Structure and concept of the interior

- Construction

- Prefabrication and Economy

Introduction

Caused by the steady social and cultural change of todays` society, demands on `living´ -as it has been known- are changing.

A new definition of “living“ needs to be offered to every person. A definition that allows a high-level living comfort freed from the steadily changing surroundings as well as external influences.

By asking for flexibility (in a building), peoples´ rapidly changing living and social habits are being well reflected.

Examples are residential relocations which are due to the present job market on the one hand as well as the free choice of working locations due to a European wide opening of the boarders.

The living-box is designed for one or two inhabitants of any age.

Transport

In order to be effortless to handle for anyone the transportation system of the living-box ought to be as easy as possible.

The only necessity to fulfil this demand is a truck with a trailer.

The living-box consists of two modules which can be linked. In the transport-mode they correspond to usual measurements of containers. The whole furniture is integrated into four inboarded cubes (two per each module).

Any personal items will be placed inside the cubes during transportation. An expensive move, as it is done today, is no more necessary.

Positioned properly, four hydraulic cylinders can be extended independently in order to level out topographic differences up to 0,5m.

Concept of living

After positioning the two basic modules in one of the three possible variations, all cubes will lower hydraulically and will release the now uprising area as the main living space, which can be personally defined by modifying (pulling / pushing out; folding out) the interior furniture. Thereby the living floor space is being increased from 26m² (transport-mode) up to 55m².

The basic modules are linked by folding out the facade (left and right in module one and up and down in module two) and build up a small connecting passage. The entry of the living-box is being generated by folding out another element of the facade in place of a usual door.

All in all there are three varieties of combining the basic modules in order to ensure adjustability for any fictitious location of the living-box. The aim is to leave as much freedom as possible to the owners´ decision of when and where he/she wants to move and which configuration is most comfortable for him/her.

Structure and Concept of the Interior

Any fixed installations - furniture as well as sanitary facilities – are integrated into the four extendible cubes. This space can be modified by extendable, flippable and multifunctional elements and combines the cubes with the basic-modules into one living-unit.

A living- and working area, as well as the sleeping area and a kitchen and bath form four integrated areas:

Living and working

Starting with the living- and working area one can experience the multifunctional furniture. The expandable seating arrangement can be changed into a sculpture of furniture and additional seating arrangements. While being in the pulled-out position it can also be used as a working place with a desk. The shelves on the wall across the room can be easily turned into a multimedia wall by turning them around.

Sleeping

The sleeping-area is separated from the rest of the living-area by a closed wardrobe. In the opened position one can use this wall as a wardrobe and the bed.

Kitchen

The center of the kitchen forms a cookingisland out of which a table for four people can be pulled out. It also offers enough space to store four stools.

Bath

The cube has hybrid attributes to it. On the one hand it is used as an expandable wardrobe and on the other hand as a fully equipped sanitarycore with a washing stand, toilet, shower and a built-in wardrobe.

Construction

The construction of the elements exists of a steel light (-weight) frame. In order to reduce the constructionhight a vacuuminsulation is being used.

The façade consists of coloured metal sheet.

The four expandable cubes are being moved by four hydraulic cylinders which are connected to the main construction.

Prefabrication & Economy

The Living-Box can be entirely prefabricated in single boxes. This guarantees an all-weather mounting and also a production of the highest quality all year long.

Even the production of smaller amounts makes economic sense due to the choice of measurement and kind of construction of the modules.